Phase 1: Creating the "Birth Certificate" in AWS

- Log into the AWS Console and search for IoT Core.

- Create the Thing:

  - On the left, go to All devices > Things.

  - Click Create things > Create single thing.

  - Thing name: Heater_Main_01.

  - Keep everything else as default and click Next.

- Configure Certificates:

  - Select Auto-generate a new certificate (recommended). Click Next.

- Policies (The "Driver's License"):

  - You likely don't have a policy yet. Click Create policy.

  - Policy name: Heater_Policy.

  - Policy statements: For now, to make sure we don't get blocked, use the "Simple Mode" or JSON tab to allow everything:

    - Action: iot:*

    - Resource: *

    - Effect: Allow

  - Click Create, then go back to your Thing tab, select this new policy, and click Create thing.

Phase 2: The "Golden Files" (Download Now)

A pop-up will appear. Stop! You must download these files now, or they are lost forever:

- Device Certificate (...certificate.pem.crt)

- Private Key file (...private.pem.key) — This is the most important one.

- RSA 2048 bit key: Amazon Root CA 1 (The Root CA).

| Item | Amazon Root CA 1 | Device Certificate & Private Key |
| --- | --- | --- |
| Quantity | ONE for everyone. | UNIQUE for every board. |
| Purpose | Proves AWS is real. | Proves the Board is real. |
| Analogy | The "Address" of the bank. | The "Key" to your specific safe. |
| Security | Public knowledge. | Top Secret. |

Phase 3: Uploading the Root CA to the Arduino

Before the Arduino can use your private keys, it needs to "trust" Amazon. We do this by uploading the Root CA directly to the WiFi module's storage.

- In the Arduino IDE, go to Tools > WiFi101 / WiFiNINA Firmware Updater.

- Select your Arduino R4 WiFi.

- Look for the section "Upload SSL Root Certificates".

- Click "Add Website" and type iot.eu-central-1.amazonaws.com (or your specific region).

- Click Upload Certificates.

  - This "burns" the trust into the ESP32-S3 bridge so it won't reject the AWS connection.

Phase 4: The "Secrets" Preparation

Open your Arduino IDE and create a new tab called arduino_secrets.h. Paste this template:

C++

#define SECRET_SSID "YOUR_WIFI_NAME"

#define SECRET_PASS "YOUR_WIFI_PASSWORD"

// Find this in AWS IoT Core > Settings > Device data endpoint

#define SECRET_BROKER "xxxxxxxxxxxxxx-ats.iot.eu-central-1.amazonaws.com"

const char SECRET_CERTIFICATE[] = R"(

-----BEGIN CERTIFICATE-----

[PASTE CONTENT OF YOUR ...certificate.pem.crt HERE]

-----END CERTIFICATE-----

)";

const char SECRET_PRIVATE_KEY[] = R"(

-----BEGIN RSA PRIVATE KEY-----

[PASTE CONTENT OF YOUR ...private.pem.key HERE]

-----END RSA PRIVATE KEY-----

)";

Let’s strip everything back to the basics. Imagine AWS IoT Core is a highly secure building and your Arduino is a remote employee trying to send a report to the main office.

Here is the step-by-step breakdown of the "Architecture" we built today.

1. The "Thing" (The Identity)

In AWS, a Thing is just a digital file that says: "I recognize there is a device out there called Heater_Arduino_Main."

- Why we do it: Without a "Thing" entry, AWS doesn't know who to attribute the data to. It’s like a name tag in a system.

- Arduino View: The Arduino must use this exact string as its clientId. If the Arduino says its name is "Heater1" but the Thing is named "Heater_Arduino_Main," AWS shuts the door.

2. The "Policy" (The Permission)

Think of the Policy as a Security Badge. Even if you have an ID, you need a badge to enter specific rooms.

- Why we do it: By default, AWS allows zero actions. The policy we created (the JSON code) explicitly says: "Anyone holding this badge is allowed to Connect, Publish (send data), and Subscribe (receive data)."

- The "Resource" Rule: We set the resource to *, which means the badge works for any "topic" (mailbox) in your AWS account.

3. The Certificates (The Passport)

This is the most complex part. AWS uses Mutual Authentication (mTLS). Instead of a password, the Arduino and AWS show each other "Passports" to prove they are who they say they are.

There are 3 Passports involved:

- The Root CA (The Government): This proves that AWS is actually Amazon. The Arduino checks this first to make sure it isn't talking to a hacker’s fake server.

- The Device Certificate (The Passport): This is the Arduino’s ID. It is signed by AWS, so AWS trusts it.

- The Private Key (The Fingerprint): This stays only on the Arduino. It is used to "sign" every message. AWS uses the Certificate to verify that the signature matches the fingerprint.

4. The Communication Flow (The Handshake)

When you see "Attempting Secure Connection" on your Serial Monitor, here is what is happening behind the scenes:

- TCP Connection: The Arduino calls the AWS IP address. AWS picks up the phone.

- Server Hello: AWS sends its Root CA to the Arduino. The Arduino checks its SECRET_CA to see if it recognizes "Amazon." (This is why having the right Root CA 3 is vital).

- Client Hello: AWS says, "Now prove who you are." The Arduino tries to send the Device Certificate.

- The Crash (Error -2): This is where we got stuck. Because the "Passport" (Certificate) was too long (RSA), the Arduino couldn't finish "handing it over" to AWS. Its memory filled up, and it hung up the phone.

- Success (The Goal): If the certificate is small (ECC), AWS reads it, verifies the signature with the Private Key, and says: "Identity confirmed. You may now send your heater temperature."

5. Summary of Our Actions Today

| Action | Why we did it | Result |
| --- | --- | --- |
| Switched to ECC | To make the "Passport" smaller. | Partially worked; AWS kept sending "Long" versions. |
| Created Policy | To give the Arduino permission to talk. | Success. Policy is ready and waiting. |
| Set up Logging | To see why AWS was rejecting us. | Success. We saw your console disconnecting. |
| Syncing Time | Encryption math relies on the exact second. | Success. Arduino knows the date/time. |

The "Why" of the Key Mismatch

The reason we struggled today is that we were trying to give the Arduino an ECC Fingerprint (Private Key) but a Passport (Certificate) wrapped in RSA paper. The Arduino got confused because the math didn't match, and the "RSA paper" was too heavy for its memory to carry.

Tomorrow, we will ensure the Passport and the Fingerprint are both ECC, which is the "lightweight language" the Arduino R4 speaks fluently.

Does this high-level view help you see how the Thing, Policy, and Certificate all "plug" into each other?