Requirement for heater project

Activate home heating

Req 1 :

A calibration must be added in the html interface to activate the heating, if this calibration is set to zero, the water pump and the water valve relays are set to not active

Req 2 :

if the temperature of the home is below the setpoint temperature of the home + 1 °C, the control of the home temperature is activated, there is a hysteresis of 0.8°C also, the control is deactivated above this threshold and reactivated 0.2 °C above the set point

Req 3 :

If the control of the home temperature is activated, the water pump is on and the control of the water valve is activated

Req 3 :

the control of the water vavle is done via a PI controller.

Req 4 :

The I of the integrator is always initialized at 0

Req 5 :

At system restart, and before any control of the water valve, the water valve shall be set to 1 and wait 150 s before the control of the PI is activated, we want to be sure that the valve is completely open,

Req 6:

the output of the PI controller shall be always between 0 and 1